Indian Institute of Technology Hyderabad

Department of Electrical Engineering

Two-Stage Operational Transconductance Amplifier (OTA) Design

45 nm CMOS Technology (GPDK045) — Cadence Virtuoso / Spectre

Submitted by

Nimmala Sree Prabhath

Roll No: EE25MTECH11010

MTech (MVLSI)

Indian Institute of Technology Hyderabad

# Abstract

This report documents the design and Cadence Spectre-based characterization of a two-stage Miller-compensated Operational Transconductance Amplifier (OTA) in 45 nm CMOS technology (GPDK045), implemented using I/O (thick-oxide) devices to safely support a 1.8 V supply. The design (L = 500 nm, 20 µA tail current) was verified through DC operating-point, AC (gain/phase), and transient simulations, with particular attention to open-loop gain, unity-gain bandwidth, and phase margin. AC characterization was performed at two input common-mode operating points — Vcm = 1.6 V and Vcm = 800 mV — to verify consistency of the small-signal response across the input common-mode range. Both bias points show a low-frequency gain of ≈48.88 dB, a unity-gain bandwidth of ≈29.65 MHz, and a measured phase margin of ≈51.17°, comfortably exceeding the 45° target and confirming that the Miller compensation network provides adequate stability margin for the intended application.

# Table of Contents

1. Objective and Specifications	4

2. Circuit Schematic	4

3. AC Characterization at Vcm = 1.6 V	4

4. AC Characterization at Vcm = 800 mV	6

5. Summary	7

# 1. Objective and Specifications

The objective of this project was to design and characterize a two-stage Miller-compensated OTA in 45nm CMOS (GPDK045) using I/O (thick-oxide) devices to safely support a 1.8V supply. The design was verified through DC operating-point, AC (gain/phase), and transient simulations in Cadence Spectre, with particular attention to open-loop gain, unity-gain bandwidth, and phase margin at two input common-mode operating points — Vcm = 1.6V and Vcm = 800mV — to check consistency of the AC response across the input common-mode range.

| Parameter | Specification |
| --- | --- |
| Technology Node | 45 nm CMOS (GPDK045), I/O (thick-oxide) devices |
| Topology | Two-stage OTA (Miller-compensated) |
| Supply Voltage | 1.8 V |
| Channel Length (L) | 500 nm |
| Tail Current | 60 µA |
| Miller Compensation Capacitor | 2 pF |
| Target Slew Rate | 20 V/µs |
| Target Phase Margin | Above 45° |
| Input Common-Mode Points Tested | Vcm = 1.6 V and Vcm = 800 mV |
| Simulator | Cadence Spectre (Virtuoso ADE-L) |

# 2. Circuit Schematic

The OTA consists of a PMOS-input differential pair (M0/M1) with an NMOS current-mirror load (M3/N2) forming the first stage, followed by a common-source second stage (M7) biased by a tail current source. Miller compensation is realized using C1 (series with an internal compensation resistor path) between the first and second stage output nodes, with C2 as the output load capacitor. Bias currents for the differential pair and the second stage are set by a cascoded current-mirror bias network (M4, M5, M11) referenced from a 25µA bias current source (I5).

Fig. 2.1 — Two-stage OTA schematic with differential input pair, Miller compensation, and current-mirror biasing

# 3. AC Characterization at Vcm = 1.6 V

The first test point places the differential input common-mode voltage at Vcm = 1.6V, close to the positive supply rail, to check the OTA's gain and phase behavior near the upper end of its input common-mode range.

### ADE Window — Setup

Three analyses were configured together: a DC operating-point analysis, an AC sweep from 100Hz to 100MHz for gain/phase extraction, and a transient analysis (0 to 1µs) for slew-rate characterization. Vcm was set to 1.6V as a design variable alongside the differential-pair bias variables x and y.

Fig. 3.1 — ADE-L setup: DC/AC/Transient analyses configured with Vcm = 1.6V

### Operating Point Annotation

With the DC operating point solved, Cadence's annotate-OP-values feature was used to back-annotate Id, Vgs, Vds, gm and region for every transistor directly on the schematic, confirming all devices are correctly biased in saturation (region=2) for this common-mode setting.

Fig. 3.2 — Schematic with annotated DC operating points (Vcm = 1.6V)

### Simulation Output — AC Gain and Phase

The Bode plot shows a low-frequency (DC) gain of about 48.88dB, rolling off with a dominant pole set by the Miller compensation network. The phase starts near 180° at low frequency (as expected for this inverting/differential configuration) and falls off through the dominant and second pole. At the unity-gain crossover frequency of about 29.65MHz, the phase has fallen to about 51.17°, giving a measured phase margin of ≈51.17° at this bias point.

Fig. 3.3 — AC Gain (magenta) and Phase (green) vs. frequency, Vcm = 1.6V

| Parameter | Value |
| --- | --- |
| Low-Frequency Gain | ≈ 48.88 dB |
| Unity-Gain Bandwidth (UGB) | ≈ 29.65 MHz |
| Phase at UGB | ≈ 51.17° |
| Measured Phase Margin | ≈ 51.17° |

# 4. AC Characterization at Vcm = 800 mV

The second test point moves the common-mode voltage down to Vcm = 800mV, roughly mid-rail, to verify that the OTA's small-signal behavior stays consistent across a wider portion of the input common-mode range rather than being characterized at only one bias point.

### ADE Window — Setup

The same DC and AC analyses were re-run with Vcm changed to 800mV, while x and y (the differential-pair sizing variables) were kept unchanged so that any difference in the result can be attributed to the shift in common-mode bias rather than a change in device sizing.

Fig. 4.1 — ADE-L setup: DC/AC analyses configured with Vcm = 800mV

### Operating Point Annotation

The back-annotated operating points at this lower common-mode voltage again confirm all transistors remain in saturation, with the expected shift in individual Vgs/Vds values relative to the Vcm = 1.6V case as the input pair's headroom changes.

Fig. 4.2 — Schematic with annotated DC operating points (Vcm = 800mV)

### Simulation Output — AC Gain and Phase

The resulting Bode plot at Vcm = 800mV closely tracks the Vcm = 1.6V result — the same ≈48.88dB low-frequency gain, the same ≈29.65MHz unity-gain bandwidth, and the same ≈51.17° phase margin — indicating the OTA's small-signal AC response is essentially independent of where the input common-mode sits within this range, which is a desirable robustness property for the intended application.

Fig. 4.3 — AC Gain (magenta) and Phase (green) vs. frequency, Vcm = 800mV

# 5. Summary

- A two-stage Miller-compensated OTA was designed in 45nm CMOS (GPDK045) using I/O devices for 1.8V operation, with L=500nm and a 20µA tail current.

- AC (gain/phase) simulations were performed at two input common-mode points, Vcm = 1.6V and Vcm = 800mV, to verify consistency of the small-signal response.

- Both bias points show a low-frequency gain of =48.88dB, a unity-gain bandwidth of =29.65MHz, and a measured phase margin of =51.17°.

- The measured phase margin of =51.17° at both bias points comfortably meets the target of above 45°, confirming the Miller compensation network provides adequate stability margin for the intended application.

- DC operating-point annotation confirms all transistors remain correctly biased in saturation across both tested common-mode conditions.